Name:

Student ID:

Signature:

THE UNIVERSITY OF NEW SOUTH WALES

School of Electrical Engineering and Telecommunications

ELEC2133: Analogue Electronics

FINAL EXAMINATION

Term 2, 2023

Reading time:                      10 minutes

Time allowed:                      2 hours

Total marks:                         100

Total number of questions:   3

This exam contributes to 50% of the total assessment of this course.

The exam contains three questions and has nine pages.

Part marks for all questions are indicated.

The candidates must attempt all the questions.

The exam paper must not be retained by the candidate and returned with the booklets.

Use separate answer booklets for each question.

The examination is closed book.

The candidates are allowed to bring UNSW approved calculators.

Assumptions made in answering the questions must be stated explicitly.

All answers must be written in inks, except where they are expressly required to use pencil for drawing, sketching, or graphical work.

QUESTION 1 [25 marks]

- A monostable multivibrator circuit is shown in Fig. 1(a). The circuit can be triggered by applying a narrow positive input pulse of height greater than Vref. For normal operation, C1R1<< CR. The saturation voltages of the op-amp are VL and –VL.

Fig. 1 (a): Monostable multivibrator

- In the stable state, what are the values of Vo, VA, and VB                                            [3 marks]

- Show the resulting waveforms of Vo and VA after the circuit has been triggered.

[4 marks]

- Show that the pulse generated at the output will have a width T given by

T=CRln2VLVref                                                            [6 marks]

- The ladder type of Digital-to-Analogue converter (DAC) is shown in Fig. 2(b). The blocks labelled Si represents a switch that connects to Vref when the bit ai=1 and ground when the bit ai=0. Calculate the output voltage, Vo, assuming an 8-bit DAC and a binary word of (10010011), R2 = 100kΩ, R1 = 10kΩ, R = 10kΩ and Vref =6V.

[6 marks]

Fig. 1 (b):  Ladder type DAC

- Assume the op-amp used in the ladder type of Digital-to-Analogue converter (DAC) shown in Fig. 1 (b) has DC imperfections and shows offset error as a result.

  - Find the expression for the offset voltage of the DAC in terms of R1, R2 and R. Assume the input bias currents are IB+ and IB-, and the input offset voltage Vio.                       [4 marks]

  - Based on your expression in (i), what value of R would you choose to minimize the offset?

[2 marks]

QUESTION 2 [35 marks]

- Four amplifier circuits are shown in Fig. 2(a)-(d). The amplifiers are biased by a current source established by a MOSFET current mirror pair Q3 and Q4.  Transistor aspect ratios (W/L) of the MOSFETs are indicated in the figure. Details of small signal model parameters are given in the information sheet on page 3.

(a)                                                                                          (b)

(c)                                                                           (d)

Fig. 2(a) –(d): Amplifier Circuits

- Identify the configuration of each of the transistors in the amplifiers in Fig. 2(a)-(d).

[2 marks]

- Compare the amplifier circuits above in terms of gain, bandwidth, and output impedance without doing calculations and provide justification for your comparison.                                      [4 marks]

- A differential amplifier circuit is shown in Fig. 2 (e). The amplifier is biased by a current source established by a BJT current mirror pair Q3 and Q4.  Details of small signal model parameters for the transistors are given in the information sheet on page 3.

Fig. 2(e): Differential amplifier circuits

- Determine the current in the current source established by the BJT current mirror in the differential amplifier shown in Fig. 2(e).                                                                                              [3 marks]

- Determine the DC biasing current in Q1 and Q2 in Fig. 2(e).                                              [2 marks]

- Calculate the gm1 rπ1 of Q1 and gm2, rπ2 of Q2 in Fig. 2(e).                                                  [2 marks]

- Calculate the DC voltages at nodes w, x, y and z. Show that transistor Q1 and Q2 in Fig. 2(e) are operating in the forward active regions, respectively.                                             [5 marks]

- Calculate the gain of the amplifier (assuming the differential amplifier in Fig. 2(e) single ended – output either at Y or Z).                                                                                                      [5 marks]

- Calculate the high cut-off frequency (fH) of the differential amplifier in Fig 2(e).           [7 marks]

- Comment on the possible values of resistors that may replace 7k to ensure that the differential amplifier in Fig. 2(e) is biased appropriately. [ Note: for amplifier operation, all BJTs must operating in the forward active regions]

[5 marks]

QUESTION 3 [40 marks]

The amplifier circuit in Fig. 2(e) is now modified with feedback as shown in Fig. 3 (a). An external capacitor, Cm, is added at the output of the differential amplifier, which is applied to a difference amplifier for further amplification before the output of the circuit is fed back to the input. For this question, assume that the op-amp in the difference amplifier as ideal.

Fig. 3 (a): Amplifier with feedback

- Identify the type of feedback topology employed in the amplifier.                                    [5 marks]

- Identify and determine the β of the feedback network.                                                           [5 marks]

- Draw the open loop amplifier circuit considering the loading effect of the feedback networks.                                                                                                                            [5 marks]

- Calculate the mid-band open loop voltage gain of the amplifier.                                            [6 marks]

- Determine the closed-loop mid-band gain of the amplifier.                                                     [4 marks]

- Calculate the output (Rout) and input (Rin) resistances of the closed-loop amplifier at midband.

[5 marks]

- Calculate the fH of the feedback amplifier.                                                           [4 marks]

- Now the feedback amplifier in Fig. 3 (a) is modified to Fig. 3 (b) by connecting Y and Z to the different input terminals of the difference amplifier, comment on how the gain, bandwidth, input and output resistances may be affected?                                                                    [6 marks]

Fig. 3 (b): Amplifier with feedback

END OF EXAMINATION